August 2015

Lifting device for special applications

Suction solution from euroTECH handles glass panes with extreme structured surfaces

The specially for glass processor Sedak designed euroTECH vacuum lifting device is capable of handling three-tonne glass segments of 14 metres in width, three metres in height and 10 millimetres thickness at a time. The weight and size of the segments are remarkable on their own, but the panes being handled also exhibit extremely structured, rather than smooth, surfaces. The suction cups are each fitted with a sealing ring of special chloroprene, allowing them to form an air-tight connection to the component in spite of the uneven surface. This innovative handling solution from euroTECH can move the facade elements from a horizontal position to a vertical transport position both safely and reliably. 1,800 of these segments will soon adorn the new headquarters of a major American electronics manufacturer as a means of solar protection.

"We didn't think that suction technology would be able to cope with these large, heavy and uneven facade elements until euroTECH showed us otherwise," explains Andreas Wenninger, Head of Applications Engineering at Sedak. The plastic adhesive on the glass combined with metal panels to aid assembly and stability give rise to a nap structure across the entire surface of the glass, which is highly unfavourable for conventional vacuum technology. A mechanical solution to the problem was initially proposed, but the vacuum application from euroTECH proved a far more practical and ultimately cost-efficient option for Sedak.

The specially designed handling solution can pivot up to 105 degrees and uses 30 adjustable, flexible and spring-loaded suction cups arranged along the cross beams of a lightweight support frame. The suction cups can be controlled individually by hand or from a central operating console. Each suction cup has a maximum load lifting capacity of 2,300 N at 60% vacuum.

The vacuum is provided by three monitored vacuum circuits. The device satisfies all of the requirements of DIN EN 13155. A warning lamp ignites and an acoustic signal is heard if the device is switched to suction mode when one of the vacuum circuits is not yet ready. In the event of a power failure, the non return valve is capable of maintaining the under pressure for at least 5 minutes, which is ample time for the loads to be lowered safely.

The lifting device was newly developed for this specific project. The modular concept used by euroTECH means that once the project has been completed, the lifting device can be adapted to meet new requirements quickly and cost-efficiently.

Company profile:

euroTECH provides handling and transport solutions in the area of vacuum technology. The company develops customer-specific vacuum systems and components for automated handling tasks. The euroTECH modular construction system enables flexible adjustment of the components to the corresponding customer requirements and fast and cost-saving exchange of replacement parts.

Attached image:

Picture

euroTECH lifting device with a three metres wide and 14 metres long glass segment

The enclosed picture material can be used by you free of charge.

Please send us a copy after the publishing.